Cosine CFM-ID Match: A Weighted Cosine Similarity-Based Workflow for MS² Spectral Matching

1. Workflow Overview

The objective of this workflow is to calculate the Weighted cosine similarity score between experimental data (measured via HRMS) and predicted data (from CFM-ID platform). The process involves several stages, including data extraction, preprocessing, normalization, m/z matching, and final cosine similarity score computation. Only compounds with a FISh score ≥ 50 are considered for experimentation.

2. Data Extraction

Step 1: Extract Data from CFM-ID Predicted Data

- Input: The predicted data is extracted from the file (p.csv), which contains m/z and intensity values

- Process:

  - Extract the m/z, intensity for each compound. These are stored in the columns mz and intensity, respectively  The CE column will be deleted

  - Normalize the mz/intensity values (0-100)

  - Round the mz and intensity values to 2 decimal places.

  - Keep only the mz values having intensity greater than 1

  - Ensure intensity values are numeric and handle any invalid or missing values.

  - Save the extracted data to a CSV file for further processing.

  - File paths: Input  dynamically;       Output  OneDrive

Step 2: Extract Experimental Data

- Input: The experimental data is stored in a separate file (x.csv), containing m/z and intensity values from the experimental measurements.

- Process:

  - Extract the m/z and intensity values, ensuring they are numeric.

  - Normalize the intensity values to a 0-100 scale by dividing by the maximum intensity in the dataset.

  - Round the m/z and intensity values to 2 decimals to ensure consistency.

  - Keep only the mz values having intensity greater than 1

  - Save the normalized data to a new CSV file, ensuring it is ready for comparison.

  - File paths: Input  dynamically;       Output  OneDrive

3. Data Preprocessing

Read CSV Files

- Load the experimental and predicted datasets.

- Ensure each row contains valid m/z and intensity values.

- Handle missing or non-numeric values with appropriate error messages.

Step 3: Check for Data Integrity

- Validate that both datasets contain valid m/z and intensity values.

- If any file is empty or contains invalid data, halt the workflow with an error message.

Step 4: Prepare Data for Comparison

- m/z Alignment: Round the m/z values to 2 decimals to ensure consistency when comparing the experimental and predicted datasets.

- Matching m/z values: define a tolerance range (e.g., ±0.01) to match m/z values between experimental and predicted datasets. This tolerance determines how close the m/z values must be considered a valid match.

4. Calculation of Weighted Cosine Similarity Score

Step 5: Identify m/z Matching

- For each experimental m/z, find the closest predicted m/z within the specified tolerance.

- If a match is found, store the experimental and predicted intensity values.

- If no match is found for a specific m/z, it is skipped for similarity calculation.

Step 7: Handling Unmatched m/z Peaks

- Define Intensity Threshold:

  - Unmatched experimental peaks with intensity ≥10% of the max intensity are included.

  - Unmatched predicted peaks with intensity ≥10% of the max intensity are included.

- Process:

  - If an unmatched experimental m/z exceeds the threshold, include it with a predicted intensity of zero.

  - If an unmatched predicted m/z exceeds the threshold, include it with an experimental intensity of zero.

Step 8: Weighted Cosine Similarity Calculation

- Compute the weighted cosine similarity between experimental and predicted intensity values.

- Formula:

Where:

Iexp,i = normalized experimental intensity.

Ipred,i = normalized predicted intensity.

wi = Weight factor assigned to each m/z peak

Step 9: Handling No Matches

- If no matches are found, set weighted cosine similarity to 0.0

Step 10: Store and Select Weighted Cosine Similarity

- Popping up weighted cosine similarity score

Visualization & Final Output

Step 11: Mirror Plot for Matched and Unmatched Peaks

- A mirror plot is generated to visually compare the experimental and predicted spectra.

Color Coding:

✅ Matched peaks → Green

❌ Unmatched peaks → Red

- Plot Layout:

  - The experimental peaks are plotted above the baseline, and the predicted peaks are plotted below the baseline for clear comparison.

5. Final Output

- The final output:

  - The weighted cosine similarity score between experimental and predicted data.

  - The mirror plot popping up

- This score represents the most accurate match between the experimental and predicted datasets.

6. Workflow Summary

1. Data Extraction & Preprocessing

- Experimental (x.csv) and predicted (p.csv) datasets are extracted, normalized (0-100 scale), and cleaned (removal of invalid/missing values and low-intensity peaks).

- m/z values are rounded to two decimal places for consistency.

- A tolerance range (e.g., ±0.01) is applied for matching m/z values between datasets.

2. Calculation of Weighted Cosine Similarity

- m/z Matching: Experimental peaks are compared against predicted peaks within the specified tolerance.

- Handling Unmatched Peaks: Peaks with intensity ≥ 10% of max intensity are retained, even if unmatched.

- Weighted Cosine Similarity Score: Calculated using normalized intensity values and assigned weights for each m/z peak.

- If no matches are found, the similarity score is set to 0.0.

3. Visualization & Output

- Mirror Plot:

✅ Matched peaks → Green

❌ Unmatched peaks → Red

- Experimental peaks are plotted above the baseline, and predicted peaks are plotted below the baseline for a clear visual comparison.

4. Final Output:

- The best weighted cosine similarity score is displayed.

- A mirror plot is generated to validate spectral matching.

Thresholding the Cosine Similarity Score:

- High confidence identification (≥ 0.80): Strong spectral agreement, highly reliable match.

- Moderate confidence identification (0.60 – 0.80): Reasonable match but may require additional validation due to potential interference or isomeric overlap.

- Low confidence identification (< 0.60): Poor spectral agreement, suggesting the need for further investigation or alternative identification strategies.

Steps

To validate and level the compounds in levels 3.1, 2.1, and 2.2

Filter compounds in CD

Make an excel sheet with names of compounds numbering 1,2,…

Experimental spectra:

- In Compound Discoverer  Copy/Paste the MS2 as points into a txt file (name as:  x.txt)

- Save the txt file in a separate folder (e.g. x) numbering 1, e.g. 1.txt

Predicted spectra:

- Open CFM-ID  Apply SMILES of the compound in the desired polarity mode  Click download  Copy/Paste the information in the CFM-ID into a text file

- Save the txt file in a separate folder (e.g. p) numbering 1, e.g. 1.txt

Run R code_x   1.csv generated

Run R code_p   1.csv generated

Run R code_ Code_weighted_Cosine_Common&uncommonPeaks

Change the numbers for each relevant compound and do all steps for every compound